St. Lucie Estuary

Current Conditions:

Over the past week, no discharge occurred at S-80. Discharge from the Lake at S-308 averaged 345 cfs.  Provisional data indicates that no discharge occurred at S-97 on C-23 and S-49 on C-24.  The current weekly average salinities (in bold) at the four monitoring sites in the St. Lucie are given below in parts per thousand (ppt), along with the previous week’s (in parenthesis).

|  | Weekly Average Salinity (ppt) |  |  |
| --- | --- | --- | --- |
| Sampling Site | Surface | Bottom | Envelope |
| Palm City Bridge (S. Fork) | 20.7 (20.5) | 21.9 (21.5) |  |
| HR1 (N. Fork) | 25.3 (24.8) | 26.3 (25.7) |  |
| Roosevelt Bridge | 27.6 (27.3) | 27.6 (27.5) | 8.0 – 25.0 |
| A1A Bridge | 31.3 (31.6) | 33.2 (33.4) | 20.0 – 31.0 |

Salinity throughout the estuary increased slightly last week, except at A1A Bridge which had a slight decrease.  At both the Roosevelt and A1A Bridges, weekly average salinity is slightly above the upper bound of the preferred range.  Salinity conditions in the estuary are considered to be fair considering the time of year, the location in the estuary, and salinity preference of the oyster, Crassostrea virginica.

Caloosahatchee Estuary

Current Conditions:

During the past week, no discharge occurred at S-79. Discharge averaged 128 cfs at S-78, while discharge at S-77 was unavailable.  The concentration of chlorides at the Olga Plant was 157 ppm on Sunday.  The current weekly average salinities (in bold) at the six monitoring sites in the Caloosahatchee Estuary are given below in parts per thousand (ppt), along with the previous week’s (in parenthesis).

| Weekly Average Salinity (ppt) |  |  |
| --- | --- | --- |
| Sampling Site | Surface | Bottom |
| Franklin Locks (S-79) | NR (5.3) | 9.1 (6.4) |
| Rt. 31 Bridge | 8.5 (5.3) | 9.1 (7.0) |
| I-75 Bridge | 9.6 (6.0) | 11.4 (9.6) |
| Ft. Myers Yacht Basin | 17.1 (14.8) | 17.9 (18.5) |
| Cape Coral Bridge | 24.7 (24.7) | 25.6 (25.2) |
| Shell Point | 34.7 (34.1) | 34.8 (34.5) |

Average salinity increased throughout the estuary.  Conditions in the upper estuary east of Ft. Myers are poor (30 day average at Ft. Myers = 17.2 ppt). Salinities at the Cape Coral Bridge are within the preferred range for the oyster, Crassostrea virginica, and well within the range tolerated by seagrass, Halodule wrightii.  Therefore, conditions in the lower estuary are good.  Salinity at Shell Point indicates that conditions are good in San Carlos Bay.

FWRI (Fish and Wildlife Research Institute) reports that water samples collected alongshore between Pinellas and Monroe counties contained no K. brevis (www.floridamarine.org).  Monitoring data collected by the River, Estuary and Coastal Observing Network (RECON) of Sanibel-Captiva Conservation Foundation (SCCF) indicated DO concentration at Ft. Myers and Shell Point ranged from 5.2 to 7.5 mg/L and chlorophyll at Ft. Myers  and Shell Point ranged from 1.0 to 16.0 mg/L.  (www.recon.sccf.org).

https://my.sfwmd.gov/portal/page?_pageid=1314,2554645,1314_19738269:1314_19738234&_dad=portal&_schema=PORTAL

click on “Current Week”

click on “Technical Summary”

scroll to the bottom and click on “Coastal Ecosystems”